# APPENDIX E — MATH / LOWER BOUND CALCULATIONS

USP Content Uniformity and Dissolution Tests

Content Uniformity (USP <905>)

Stage 1) Test 10 dosage units. Requirements are met if the acceptance value of the first 10 units satisfies

$$AV \le L_1 = 15$$

Otherwise go to Stage 2.

Stage 2) Test an additional 20 units. Pass if, for all 30 units:

$$AV_{30} \le 15 \quad\text{and}\quad |x_i - M| \le 0.25\,M \;\; \forall i$$

The acceptance value is defined as

$$AV = |M - \bar{X}| + k\,s, \qquad k = \begin{cases} 2.4 & n = 10 \\ 2.0 & n = 30 \end{cases}$$

where $\bar{X}$ is the sample mean and $s$ the sample standard deviation. $M$ is based on the target $T$ (average of the monograph potency limits, % of label claim):

For $T \le 101.5$:

$$M = \begin{cases} \max(98.5,\ \bar{X}) & \text{if } \bar{X} \le 100 \\ \min(101.5,\ \bar{X}) & \text{if } \bar{X} > 100 \end{cases}$$

For $T > 101.5$:

$$M = \begin{cases} \max(98.5,\ \bar{X}) & \text{if } \bar{X} \le 100 \\ \min(T,\ \bar{X}) & \text{if } \bar{X} > 100 \end{cases}$$

Dissolution (USP <711>)

Stage 1) Test 6 units. Pass if

$$x_i \ge Q + 5 \quad \forall\, i = 1,\dots,6$$

Stage 2) Test 6 additional units. Pass if

$$\bar{x}_{12} \ge Q \quad\text{and}\quad x_i > Q - 15 \quad \forall i$$

Stage 3) Test 12 additional units. Pass if

$$\bar{x}_{24} \ge Q,$$

with no more than two results $\le Q - 15$ and no result $\le Q - 25$. Otherwise Fail.

Statistical Methods

Assume normal dosage-unit results $X_i \sim \mathcal{N}(\mu, \sigma^2)$. Then

$$Z_1 = \bar{X} \sim \mathcal{N}\!\left(\mu, \tfrac{\sigma}{\sqrt{n}}\right), \qquad Z_2 = \frac{(n-1)s^2}{\sigma^2} \sim \chi^2_{n-1}$$

are independent, and the overall lower bound of passing the USP test is

$$P(\text{pass}) \ge \max\{P(S_1),\, P(S_2)\}$$

Computation of $P(S_1)$

With $L_1 = 15$ and $k_1 = 2.4$:

$$P(S_1) = \big(\Phi(t_1) - \Phi(t_2)\big)\; P\!\left(\chi^2_{n-1} < \frac{(n-1)L_1^2}{k_1^2\,\sigma^2}\right) + I_2 + I_3$$

$$t_1 = \frac{101.5 - \mu}{\sigma/\sqrt{n}}, \qquad t_2 = \frac{98.5 - \mu}{\sigma/\sqrt{n}}$$

The tail integrals are evaluated numerically with step $h = 0.05$:

$$I_2 = \sum_{i=1}^{K} \big[\Phi(z_0 + ih) - \Phi(z_0 + (i-1)h)\big]\; P\!\big(\chi^2_{n-1} < g(z_0 + (i-\tfrac{1}{2})h)\big)$$

$$g(z_1) = \frac{(n-1)\,\big(L_1 + 101.5 - z_1\big)^2}{k_1^2\,\sigma^2}$$

$$I_3 = \sum_{i=1}^{K} \big[\Phi(z_0 + ih) - \Phi(z_0 + (i-1)h)\big]\; P\!\big(\chi^2_{n-1} < q(z_0 + (i-\tfrac{1}{2})h)\big)$$

$$q(z_1) = \frac{(n-1)\,\big(L_1 - 98.5 + z_1\big)^2}{k_1^2\,\sigma^2}$$

Computation of $P(S_2)$

Let $C_{21}$ = “AV of the 30 units $\le L_1$” and $C_{22}$ = “no unit deviates from $M$ by more than $L_2 = 25$”. Using $P(A \cap B) \ge P(A) + P(B) - 1$:

$$P(S_2) = P(C_{21} \cap C_{22}) \ge \max\{P(C_{21}) + P(C_{22}) - 1,\; 0\}$$

$P(C_{21})$ is computed exactly like $P(S_1)$ with $n = 30$, $k_2 = 2.0$, and

$$P(C_{22}) \ge \left[ \Phi\!\left(\frac{98.5 + L_2 - \mu}{\sigma/\sqrt{n}}\right) - \Phi\!\left(\frac{101.5 - L_2 - \mu}{\sigma/\sqrt{n}}\right) \right]^{n}$$

(replace $101.5$ by $T$ when $T > 101.5$).

Dissolution stage probabilities

Let $\ell$ be the adjusted lower bound on the mean offset from $Q$

($\ell = (\text{mean} - Q) - z\,SE_{\text{mean}}$); units are

$x \sim \mathcal{N}(Q + \ell, \sigma)$. The three stage bounds are

$$F_1 = \big[P(x \ge Q+5)\big]^6 = \left[1 - \Phi\!\left(\frac{5 - \ell}{\sigma}\right)\right]^6$$

$$F_2 = \big[P(x \ge Q-15)\big]^{12} - P(\bar{x}_{12} < Q)$$

$$F_3 = p_3^{24} + 24\,p_2\,p_3^{23} + 276\,p_2^2\,p_3^{22} - P(\bar{x}_{24} < Q)$$

with

$$p_2 = \Phi\!\left(\frac{-15 - \ell}{\sigma}\right) - \Phi\!\left(\frac{-25 - \ell}{\sigma}\right), \qquad p_3 = 1 - \Phi\!\left(\frac{-15 - \ell}{\sigma}\right)$$

$$P(\bar{x}_n < Q) = \Phi\!\left(\frac{\sqrt{n}\,(-\ell)}{\sigma}\right)$$

and the overall bound is $P(\text{pass}) = \max(F_1, F_2, F_3)$.

Plan 2 variance components

With $n$ units per location, $L$ locations and $N = nL$:

$$\chi^2_{err} = F^{-1}_{\chi^2}\!\big(1-\sqrt{C/100};\; L(n-1)\big), \qquad \chi^2_{loc} = F^{-1}_{\chi^2}\!\big(1-\sqrt{C/100};\; L-1\big)$$

$$h_1 = \frac{L-1}{\chi^2_{loc}} - 1, \qquad h_2 = \frac{L(n-1)}{\chi^2_{err}} - 1$$

$$\mathrm{VAR} = \left(s_m^2 + \left(1-\tfrac{1}{n}\right)s_e^2\right) + \sqrt{\left(h_1 s_m^2\right)^2 + \left(\left(1-\tfrac{1}{n}\right)h_2 s_e^2\right)^2}$$

$$\mathrm{MVAR} = \frac{(L-1)\,n\,s_m^2}{\chi^2_{loc}}$$

The mean limits use the standard error $\sqrt{\mathrm{MVAR}/N}$ with the confidence multipliers

$$z = \Phi^{-1}\!\left(\frac{1+\sqrt{C/100}}{2}\right) \;\; \text{(CU, two-sided)}, \qquad z = \Phi^{-1}\!\left(\sqrt{C/100}\right) \;\; \text{(dissolution, one-sided)}$$